# PROJECT REPORT

# Quantum Circuit Probability Display Using Arduino

Submitted by: Sudipta Karmakar

Sec: A1 | Roll No: 28 1st Year, BSH Department

Enrollment No: 12025052018043

Under the guidance of: Ranbir Banik

Assistant HOD (BSH Department)

Institute of Engineering and Management (Saltlake)

# Introduction

The project demonstrates a quantum-interference-inspired optical setup using an Arduino UNO, a laser, a beam splitter, two LM393 LDR sensor modules, and a 16x2 I2C LCD display. Although it does not perform real quantum computation, it simulates probabilistic behavior similar to quantum superposition using light detection and statistical sampling.

# Objective

To build an optical–electronic system that detects two light paths and displays the probability of states |0> and |1>, while indicating 'SUPERPOS' when both paths are present. The laser is controlled by Arduino pin D8 and starts after a 2-second delay.

# Components Used with Approximate Price (India)

| Component | Quantity | Approx. Price (INR) |
| --- | --- | --- |
| Arduino UNO | 1 | ■450 – ■650 |
| 16x2 I2C LCD | 1 | ■350 – ■450 |
| Laser Module | 1 | ■120 – ■180 |
| Beam Splitter | 1 | ■1600 – ■2500 |
| LM393 LDR Module | 2 | ■120 – ■180 each |
| Breadboard | 1 | ■120 – ■180 |
| Jumper Wires | 1 pack | ■80 – ■150 |
| USB Cable / Power | 1 | ■80 – ■120 |

Estimated Total Cost

Approximately ■3000 – ■4600 depending on local market and quality of components. This increase is mainly due to the higher-cost beam splitter.

# Block Diagram

Laser (D8)  Beam Splitter  Two light paths  LM393 Sensors (D2 & D7)  Arduino UNO 

I2C LCD.

# Circuit Connections

- Laser VCC  D8, Laser GND  GND - LM393 #1 DO  D2, VCC  5V, GND  GND - LM393 #2 DO  D7, VCC  5V, GND  GND - LCD SDA  A4, SCL  A5, VCC  5V, GND  GND

# Working Principle

The beam splitter divides the laser into two paths. Each path falls on an LDR sensor. The Arduino repeatedly reads both sensors, counts detections, and calculates probabilities. If both sensors detect light, interference-like behavior is labeled 'SUPERPOS'. Otherwise, only probabilities are shown.

# Display Output Explanation

The LCD screen shows two probabilities side-by-side: P(0) and P(1). If only one sensor detects light, the Arduino calculates the percentage of detection for each path based on repeated readings and displays them as percentages. If both sensors detect light simultaneously, the system assumes a superposition-like condition and displays the message 'SUPERPOS' instead of numerical probabilities, indicating that both light paths are active at the same time.

# Result & Conclusion

The project successfully demonstrates light-based probabilistic detection using Arduino and provides a clear visual output on the LCD display.

# Acknowledgement

I sincerely thank Mr. Ranbir Banik and the BSH Department of IEM Saltlake for their guidance and support in completing this project.